PH: 800-403-0011 / FX: 800-952-8281

## ANTI-FREEZE & COOLANT SPECIFICATION SHEET

### EXTENDED LIFE (RED) ANTI-FREEZE / COOLANT

READY-TO-USE FORMULA  –340F

- Unsurpassed freeze-up and boil-over protection

- Extended rust and corrosion protection

- Silicate & Phosphate Free

- Outstanding high temperature aluminum protection

- Excellent heat transfer properties

- Extends water pump life

- Compatible with DEXCOOLTM Extended Life Anti-Freeze / Coolant

- Compatible with all conventional & extended life anti-freeze

- 5 year / 150,000 mile protection when cooling system is completely flushed and filled with DEXCOOLTM Extended Life Anti-Freeze / Coolant

- Meets ASTM D-3306 and ASTM D-4340

##### CHEMICAL PROPERTIES

###### PROPERTIES				TYPICAL VALUES			ASTM TEST METHOD

SPECIFIC GRAVITY AT 60 / 600 F	1.120-1.130			D 1122

BOILING POINT REFLUX		2700 F MIN (with 15# pressure cap)	D 1120

FOAM TEST			50 ml / 3 SEC MAX			D 1881

pH 50% VOLUME SOLUTION	7.5 MIN-10.0 MAX			D 1287

FLASH POINT COC		2500 F MIN			D     92

# TOTAL WATER, Wt %		40% MAX			D 1123

TOTAL GLYCOLS, Wt %		60% MIN				E   202

COLOR				Red

##### PROTECTION CHART

FREEZE / BOIL PROTECTION (USING A 15PSI PRESSURE CAP)

% of Cooling System	PROTECTS FROM

Capacity		Freezing down to	Boiling up to

100%		-340 F		2700 F

COOLING SYSTEM CAPACITY IN QUARTS

Quarts of Anti-Freeze Required for Protection to Temperatures (F) Shown

QUARTS     6       9      12     15     18    21     24

12	    0     -34    -51

14	    6     -18    -41

16	   10    -12    -34    -47

18	   14      0     -21    -41   -51

20	   16      4     -12    -34   -45

22 	   18      8      -6     -28   -41   -48

24               19     10      0     -15   -34   -41    -51

11/03 	REFERENCE PART #2013